SOC LAB

- Full Adder

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a,b,cin,s,co;

void mon()

{

cout<<"Inputs: "<<a<<b<<cin<<"Output: "<<s<<co<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a<<b<<cin;

}

};

//main.cpp

#include <systemc.h>

#include "fulladder.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a,b,s,co,cin;

fulladder fa("fulladder");

driver drive("driver");

monitor mon("monitor");

fa.a(a); fa.b(b); fa.s(s); fa.cin(cin); fa.co(co);

drive.a(a); drive.b(b);drive.cin(cin);

mon.a(a); mon.b(b); mon.cin(cin); mon.s(s);mon.co(co);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a, "a");

sc_trace(Tf, b, "b");

sc_trace(Tf, cin, "cin");

sc_trace(Tf, s, "s");

sc_trace(Tf, co, "co");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//adder.h

#include <systemc.h>

SC_MODULE(fulladder)

{

sc_in<bool> a;

sc_in<bool> b;

sc_in<bool> cin;

sc_out<bool> s;

sc_out<bool> co;

void compute_fa()

{

s.write(a.read() ^ b.read() ^ cin.read());

co.write((a.read() & b.read()) | cin.read() & (a.read() ^ b.read()));

}

SC_CTOR(fulladder)

{

SC_METHOD(compute_fa);

sensitive<<a<<b<<cin;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a,b,cin;

void inputs()

{

a.write(false);

b.write(false);

cin.write(false);

wait(5, SC_NS);

a.write(true);

b.write(true);

cin.write(false);

wait(5, SC_NS);

a.write(false);

b.write(false);

cin.write(true);

wait(5, SC_NS);

a.write(true);

b.write(true);

cin.write(true);

wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a <<b<<cin;

}

};

Output:

- Half adder

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a,b,s,c;

void mon()

{

cout<<"Inputs: "<<a<<b<<"Output: "<<s<<c<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a<<b;

}

};

#include <systemc.h>

#include "halfadder.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a,b,s,c;

halfadder ha("halfadder");

driver drive("driver");

monitor mon("monitor");

ha.a(a); ha.b(b); ha.s(s); ha.c(c);

drive.a(a); drive.b(b);

mon.a(a); mon.b(b); mon.c(c); mon.s(s);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a, "a");

sc_trace(Tf, b, "b");

sc_trace(Tf, c, "c");

sc_trace(Tf, s, "s");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//halfadder.h

#include <systemc.h>

SC_MODULE(halfadder)

{

sc_in<bool> a;

sc_in<bool> b;

sc_out<bool> s;

sc_out<bool> c;

void compute_ha()

{

s.write(a.read() ^ b.read());

c.write(a.read() & b.read());

}

SC_CTOR(halfadder)

{

SC_METHOD(compute_ha);

sensitive<<a<<b;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a,b;

void inputs()

{

a.write(false);

b.write(false);

wait(5, SC_NS);

a.write(false);

b.write(true);

wait(5, SC_NS);

a.write(true);

b.write(false);

wait(5, SC_NS);

a.write(true);

b.write(true);

wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a <<b;

}

};

Output:

- Counters Ascyn

//andgate.h

#include <systemc.h>

SC_MODULE(dff)

{

sc_in<bool> d;

sc_out<bool> q;

sc_in<bool> clk;

void compute_dff()

{

q.write(d.read());

}

SC_CTOR(dff)

{

SC_METHOD(compute_dff);

sensitive<<clk.pos();

}

};

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> d,q;

sc_in<bool> clk;

void mon()

{

cout<<"Input D: "<<d<<"Output Q:= "<<q<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<clk.pos();

}

};

#include <systemc.h>

#include "dff.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> d,q;

sc_clock  testclk("testclock",10,SC_NS,0.5);

dff dff1("dff");

driver drive("driver");

monitor mon("monitor");

dff1.d(d); dff1.q(q); dff1.clk(testclk);

drive.d(d); drive.clk(testclk);

mon.d(d); mon.q(q); mon.clk(testclk);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, d, "d");

sc_trace(Tf, q, "q");

sc_trace(Tf,testclk,"testclk");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> d;

sc_in<bool> clk;

void inputs()

{

wait();

d.write(false);

wait();

d.write(true);

wait();

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<clk.pos();

}

};

Output:

- Sync Counter

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> rst,en;

sc_in<bool> clk;

void inputs()

{

rst.write(true);

wait(1,SC_NS);

rst.write(false);

en.write(true);

wait(1,SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<clk.pos()<<rst<<en;

}

};

//andgate.h

#include <systemc.h>

SC_MODULE(upcounter)

{

sc_in<bool> rst,en;

sc_out<sc_uint<8>> counter_out;

sc_in<bool> clk;

sc_uint<8>  count;

void compute_upcounter()

{

if (rst.read() == 1)

{

count =  0;

counter_out.write(count);

}

else if (en.read() == 1)

{

count = count + 1;

counter_out.write(count);

}

}

SC_CTOR(upcounter)

{

SC_METHOD(compute_upcounter);

sensitive<<clk.pos()<<rst;

}

};

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> rst,en;

sc_in<sc_uint<8>> counter_out;

sc_in<bool> clk;

void mon()

{

cout<<"Input rst: "<<rst<<"Output counter_out:= "<<counter_out<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<clk.pos()<<rst;

}

};

#include <systemc.h>

#include "sync_up.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> rst,en;

sc_signal<sc_uint<8>> counter_out;

sc_clock  testclk("testclock",2,SC_NS,0.5);

upcounter up("upcounter");

driver drive("driver");

monitor mon("monitor");

up.rst(rst); up.en(en); up.clk(testclk);up.counter_out(counter_out);

drive.rst(rst);drive.en(en); drive.clk(testclk);

mon.rst(rst); mon.en(en); mon.clk(testclk);mon.counter_out(counter_out);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, rst, "rst");

sc_trace(Tf, en, "en");

sc_trace(Tf, counter_out, "counter_out");

sc_trace(Tf,testclk,"testclk");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

Output:

- Decoder

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> d0,d1,d2,d3,en,s0,s1;

void mon()

{

cout<<"Inputs: "<<en<<s0<<s1<<"Output: "<<d0<<d1<<d2<<d3<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<en<<s0<<s1;

}

};

#include <systemc.h>

#include "decoder.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> d0,d1,d2,d3,en,s0,s1;

decoder dec1("decoder");

driver drive("driver");

monitor mon("monitor");

dec1.d0(d0); dec1.d1(d1); dec1.d2(d2); dec1.d3(d3); dec1.en(en); dec1.s0(s0); dec1.s1(s1);

drive.en(en); drive.s0(s0); drive.s1(s1);

mon.d0(d0); mon.d1(d1); mon.d2(d2); mon.d3(d3); mon.en(en); mon.s0(s0); mon.s1(s1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, s0, "s0");

sc_trace(Tf, s1, "s1");

sc_trace(Tf, en, "en");

sc_trace(Tf, d0, "d0");

sc_trace(Tf, d1, "d1");

sc_trace(Tf, d2, "d2");

sc_trace(Tf, d3, "d3");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//decoder.h

#include <systemc.h>

SC_MODULE(decoder)

{

sc_in<bool> s0;

sc_in<bool> s1;

sc_in<bool> en;

sc_out<bool> d0;

sc_out<bool> d1;

sc_out<bool> d2;

sc_out<bool> d3;

void compute_decoder()

{

if(en.read()==true){

if( (s1.read()==false) && (s0.read()==false))

{

d0.write(true);

d1.write(false);

d2.write(false);

d3.write(false);

}

else if((s1.read()==false) && (s0.read()==true))

{

d0.write(false);

d1.write(true);

d2.write(false);

d3.write(false);

}

else if( (s1.read()==true) && (s0.read()==false))

{

d0.write(false);

d1.write(false);

d2.write(true);

d3.write(false);

}

else if( (s1.read()==true) && (s0.read()==true))

{

d0.write(false);

d1.write(false);

d2.write(false);

d3.write(true);

}

}

else if(en.read()==false)

{

d0.write(false);

d1.write(false);

d2.write(false);

d3.write(false);

}

}

SC_CTOR(decoder)

{

SC_METHOD(compute_decoder);

sensitive<<s0<<s1<<en;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> s0,s1,en;

void inputs()

{

s0.write(false);

s1.write(false);

en.write(true);

wait(5, SC_NS);

s0.write(true);

s1.write(false);

en.write(true);

wait(5, SC_NS);

s0.write(false);

s1.write(true);

en.write(false);

wait(5, SC_NS);

s0.write(true);

s1.write(true);

en.write(true);

wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<s0<<s1<<en;

}

};

Output:

- Encoder

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<sc_lv<4>> d;

sc_in<sc_lv<2>> y;

void mon()

{

cout<<"Inputs: "<<d<<"Output: "<<y<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<d;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<sc_lv<4>> d;

void inputs()

{

d.write("0001");

wait(5, SC_NS);

d.write("0010");

wait(5, SC_NS);

d.write("1000");

wait(5, SC_NS);

d.write("0000");

wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<d;

}

};

//encoder.h

#include <systemc.h>

SC_MODULE(encoder)

{

sc_in<sc_lv<4>> d;

sc_out<sc_lv<2>> y;

void compute_encoder()

{

if( d.read()=="0001")

{

y.write("00");

}

else if( d.read()=="0010")

{

y.write("01");

}

else if( d.read()=="0100")

{

y.write("10");

}

else if( d.read()=="1000")

{

y.write("11");

}

else if( d.read()=="0000")

{

y.write("XX");

}

}

SC_CTOR(encoder)

{

SC_METHOD(compute_encoder);

sensitive<<d;

}

};

#include <systemc.h>

#include "encoder.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<sc_lv<4>> d;

sc_signal<sc_lv<2>> y;

encoder enc("encoder");

driver drive("driver");

monitor mon("monitor");

enc.d(d); enc.y(y);

drive.d(d);

mon.d(d); mon.y(y);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, d, "d");

sc_trace(Tf, y, "y");

/*sc_trace(Tf, en, "en");

sc_trace(Tf, d0, "d0");

sc_trace(Tf, d1, "d1");

sc_trace(Tf, d2, "d2");

sc_trace(Tf, d3, "d3");*/

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

Output:

- FIFO

#include "systemc.h"

#include "fifo.h"

int sc_main(int argc, char* argv[])

{

example_fifo ex_fifo("ex_fifo0");

sc_start(10, SC_NS);

return 0;

}

#include "systemc.h"

SC_MODULE(example_fifo) {

// Declare the FIFO

sc_fifo<int> packet_fifo;

void fiforead(void) {

int val;

for (;;){

wait(3, SC_NS);

packet_fifo.read(val);

cout<< sc_time_stamp() << ": Read " << val<<endl;

cout<< sc_time_stamp()<<"No.of Data in FIFO "<<packet_fifo.num_available()<<endl;

}

}

void fifowrite(void) {

int val= 10;

for (;;) {

wait(2, SC_NS);

val++;

packet_fifo.write(val);

cout<< sc_time_stamp() << ": wrote " << val<< "  No. of Free Slots in FIFO "<<packet_fifo.num_free()<<endl;

}

}

SC_CTOR(example_fifo) :packet_fifo(10)

{

SC_THREAD(fifowrite);

SC_THREAD(fiforead);

}

};

- D flipflop

//dff.h

#include <systemc.h>

SC_MODULE(dff)

{

sc_in<bool> d;

sc_out<bool> q;

sc_in<bool> clk;

void compute_dff()

{

q.write(d.read());

}

SC_CTOR(dff)

{

SC_METHOD(compute_dff);

sensitive<<clk.pos();

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> d;

sc_in<bool> clk;

void inputs()

{

wait();

d.write(false);

wait();

d.write(true);

wait();

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<clk.pos();

}

};

#include <systemc.h>

#include "dff.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> d,q;

sc_clock  testclk("testclock",10,SC_NS,0.5);

dff dff1("dff");

driver drive("driver");

monitor mon("monitor");

dff1.d(d); dff1.q(q); dff1.clk(testclk);

drive.d(d); drive.clk(testclk);

mon.d(d); mon.q(q); mon.clk(testclk);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, d, "d");

sc_trace(Tf, q, "q");

sc_trace(Tf,testclk,"testclk");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> d,q;

sc_in<bool> clk;

void mon()

{

cout<<"Input D: "<<d<<"Output Q:= "<<q<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<clk.pos();

}

};

Output:

- T flipflop

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> t,q;

sc_in<bool> clk;

void mon()

{

cout<<"Input T: "<<t<<"Output Q:= "<<q<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<clk.pos();

}

};

//andgate.h

#include <systemc.h>

SC_MODULE(tff)

{

sc_in<bool> t;

sc_out<bool> q;

sc_in<bool> clk;

void compute_tff()

{

q.write(!(t.read()));

}

SC_CTOR(tff)

{

SC_METHOD(compute_tff);

sensitive<<clk.pos();

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> t;

sc_in<bool> clk;

void inputs()

{

wait();

t.write(false);

wait();

t.write(true);

wait();

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<clk.pos();

}

};

#include <systemc.h>

#include "tff.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> t,q;

sc_clock  testclk("testclock",10,SC_NS,0.5);

tff tff1("tff");

driver drive("driver");

monitor mon("monitor");

tff1.t(t); tff1.q(q); tff1.clk(testclk);

drive.t(t); drive.clk(testclk);

mon.t(t); mon.q(q); mon.clk(testclk);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, t, "t");

sc_trace(Tf, q, "q");

sc_trace(Tf,testclk,"testclk");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

Output:

- FSM

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<sc_lv<4>> d;

void inputs()

{

d.write("0001");

wait(5, SC_NS);

d.write("0010");

wait(5, SC_NS);

d.write("1000");

wait(5, SC_NS);

d.write("0000");

wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<d;

}

};

case 1:

if (

in.read

next_state

= 1;out=

else{

next_state

= 2;out=

break;

case 2:

if (

in.read

next_state

= 1;out=

else{

next_state

= 0;out=

break;

default:

{

next_state

= 0;out=

break;

}

}

SC_CTOR(

fsm

SC_METHOD(

update_state

sensitive <<

clk.pos rst

SC_METHOD(

ns_logic

sensitive <<

clk.pos ()<< rst

}};

rst.write

(

in.write

(

wait();

in.write

(

wait();

in.write

(

wait();

in.write

(

wait();

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<

clk.pos

}

};

#include "

systemc.h

SC_MODULE(driver)

{

sc_out

bool rst,in

sc_in

bool clk

void inputs()

{

rst.write

(

in.write

(

wait();

rst.write

(

in.write

(

wait();

#include "

systemc.h

SC_MODULE(monitor)

{

sc_in

bool > rst , in,out

sc_in

bool > clk

void mon()

{

while (true){

cout

<< sc_time_stamp

cout

<< " rst rst.read

cout

<< " clk clk.read

cout

<< " in in.read

cout

<< " out out.read () endl

wait();

}}

SC_CTOR(monitor)

{

SC_THREAD(mon);

sensitive <<

clk.pos

}};

#include "

systemc.h

#include "

driver.h

#include "

monitor.h

#include "

fsm.h

int

sc_main int argc , char*

argv

{

sc_signal

bool > rst,in,out

sc_clock

clk ("TestClock",10,SC_NS,

fsm

f_sm state_machine

driver drive("driver");

monitor mon("monitor");

f_sm.rst

rst

f_sm.in(in);

f_sm.clk

clk

f_sm.out

(

drive.rst

rst

drive.in(in);

drive.clk

clk

mon.rst

rst

mon.in(in);

mon.out

(

mon.clk

clk

//Waveform

sc_trace_file

Tf

Tf

=

sc_create_vcd_trace_file ("traces

sc_trace

Tf , rst , rst

sc_trace

Tf , in, "

sc_trace

Tf , clk , clk

sc_trace

Tf , out, "

sc_trace

Tf , f_sm.state ," sc_start (100,SC_ sc_close_vcd_trace_file Tf); return 0;

- Mux

#include <systemc.h>

#include "mux.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a,b,c,d,f,s0,s1;

mux mux1("mux");

driver drive("driver");

monitor mon("monitor");

mux1.a(a); mux1.b(b); mux1.f(f); mux1.c(c); mux1.d(d); mux1.s0(s0); mux1.s1(s1);

drive.a(a); drive.b(b);  drive.c(c); drive.d(d); drive.s0(s0); drive.s1(s1);

mon.a(a); mon.b(b); mon.f(f); mon.c(c); mon.d(d); mon.s0(s0); mon.s1(s1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a, "a");

sc_trace(Tf, b, "b");

sc_trace(Tf, c, "c");

sc_trace(Tf, d, "d");

sc_trace(Tf, f, "f");

sc_trace(Tf, s0, "s0");

sc_trace(Tf, s1, "s1");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//mux.h

#include <systemc.h>

SC_MODULE(mux)

{

sc_in<bool> s0;

sc_in<bool> s1;

sc_in<bool> a;

sc_in<bool> b;

sc_in<bool> c;

sc_in<bool> d;

sc_out<bool> f;

void compute_mux()

{

if((s1.read()==false) && (s0.read()==false))

{

f.write(a.read());

}

else if((s1.read()==false) && (s0.read()==true))

{

f.write(b.read());

}

if((s1.read()==true) && (s0.read()==false))

{

f.write(c.read());

}

if((s1.read()==true) && (s0.read()==true))

{

f.write(d.read());

}

}

SC_CTOR(mux)

{

SC_METHOD(compute_mux);

sensitive<<s0<<s1<<a<<b<<c<<d;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a,b,c,d,s0,s1;

void inputs()

{

a.write(true);

b.write(false);

c.write(true);

d.write(false);

s0.write(false);

s1.write(false);wait(5, SC_NS);

a.write(true);

b.write(false);

c.write(true);

d.write(false);

s0.write(true);

s1.write(false);wait(5, SC_NS);

a.write(true);

b.write(false);

c.write(true);

d.write(false);

s0.write(false);

s1.write(true);wait(5, SC_NS);

a.write(true);

b.write(false);

c.write(true);

d.write(false);

s0.write(true);

s1.write(true);wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a<<b<<c<<d<<s0<<s1;

}

};

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a,b,c,d,f,s0,s1;

void mon()

{

cout<<"Inputs: "<<a<<b<<c<<d<<s0<<s1<<"Output: "<<f<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a<<b<<c<<d<<s0<<s1;

}

};

Output:

- SISO

//siso.h

#include <systemc.h>

SC_MODULE(siso)

{

sc_in<sc_logic> d;

sc_out<sc_logic> r;

sc_in<bool> clk;

sc_lv<3> temp;

void compute_siso()

{

r=temp[2];

temp[2]=temp[1];

temp[1]=temp[0];

temp[0]=d.read();

cout <<sc_time_stamp<< " Temp[2]: "<< temp[2]<< " Temp[1]: "<< temp[1] << " Temp[0]: "<< temp[0]<<endl;

}

SC_CTOR(siso)

{

SC_METHOD(compute_siso);

sensitive<<clk.pos();

}

};

#include <systemc.h>

#include "siso.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<sc_logic> d,r;

sc_clock  testclk("testclock",10,SC_NS,0.5);

siso siso1("siso");

driver drive("driver");

monitor mon("monitor");

siso1.d(d); siso1.r(r); siso1.clk(testclk);

drive.d(d); drive.clk(testclk);

mon.d(d); mon.r(r); mon.clk(testclk);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, d, "d");

sc_trace(Tf, r, "r");

sc_trace(Tf,testclk,"testclk");

sc_start(120, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<sc_logic> d,r;

sc_in<bool> clk;

void mon()

{

cout<<"Input D: "<<d<<" Output Q:= "<<r<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<clk.pos();

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<sc_logic> d;

sc_in<bool> clk;

void inputs()

{

wait();

d = sc_logic_1;

wait();

d = sc_logic_0;

wait();

d = sc_logic_1;

wait();

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<clk.pos();

}

};

Output:

- NAND

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a1, b1, c1;

void inputs()

{

a1.write(false);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(false);wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a1 <<b1 <<c1;

}

};

#include <systemc.h>

#include "nandgate.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a1, b1,c1, d1;

nandgate nagate("nandgate");

driver drive("driver");

monitor mon("monitor");

nagate.a1(a1); nagate.b1(b1); nagate.c1(c1); nagate.d1(d1);

drive.a1(a1); drive.b1(b1); drive.c1(c1);

mon.a1(a1); mon.b1(b1); mon.c1(c1); mon.d1(d1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a1, "a1");

sc_trace(Tf, b1, "b1");

sc_trace(Tf, c1, "c1");

sc_trace(Tf, d1, "d1");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a1,b1,c1,d1;

void mon()

{

cout<<"Inputs: "<<a1<<b1<<c1<<"Output: "<<d1<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a1<<b1<<c1;

}

};

//nandgate.h

#include <systemc.h>

SC_MODULE(nandgate)

{

sc_in<bool> a1;

sc_in<bool> b1;

sc_in<bool> c1;

sc_out<bool> d1;

void compute_nand()

{

d1.write(!(a1.read() && b1.read() && c1.read()));

}

SC_CTOR(nandgate)

{

SC_METHOD(compute_nand);

sensitive<<a1<<b1<<c1;

}

};

Output:

- NOR

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a1,b1,c1,d1;

void mon()

{

cout<<"Inputs: "<<a1<<b1<<c1<<"Output: "<<d1<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a1<<b1<<c1;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a1, b1, c1;

void inputs()

{

a1.write(false);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(false);wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a1 <<b1 <<c1;

}

};

#include <systemc.h>

#include "norgate.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a1, b1,c1, d1;

norgate nogate("norgate");

driver drive("driver");

monitor mon("monitor");

nogate.a1(a1); nogate.b1(b1); nogate.c1(c1); nogate.d1(d1);

drive.a1(a1); drive.b1(b1); drive.c1(c1);

mon.a1(a1); mon.b1(b1); mon.c1(c1); mon.d1(d1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a1, "a1");

sc_trace(Tf, b1, "b1");

sc_trace(Tf, c1, "c1");

sc_trace(Tf, d1, "d1");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//nogate.h

#include <systemc.h>

SC_MODULE(norgate)

{

sc_in<bool> a1;

sc_in<bool> b1;

sc_in<bool> c1;

sc_out<bool> d1;

void compute_nor()

{

d1.write(!(a1.read() || b1.read() || c1.read()));

}

SC_CTOR(norgate)

{

SC_METHOD(compute_nor);

sensitive<<a1<<b1<<c1;

}

};

Output:

- XOR

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a1,b1,c1,d1;

void mon()

{

cout<<"Inputs: "<<a1<<b1<<c1<<"Output: "<<d1<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a1<<b1<<c1;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a1, b1, c1;

void inputs()

{

a1.write(false);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(true);wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a1 <<b1 <<c1;

}

};

//xorgate.h

#include <systemc.h>

SC_MODULE(xorgate)

{

sc_in<bool> a1;

sc_in<bool> b1;

sc_in<bool> c1;

sc_out<bool> d1;

void compute_xor()

{

d1.write(a1.read() ^ b1.read() ^ c1.read());

}

SC_CTOR(xorgate)

{

SC_METHOD(compute_xor);

sensitive<<a1<<b1<<c1;

}

};

#include <systemc.h>

#include "xorgate.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a1, b1,c1, d1;

xorgate xogate("xorgate");

driver drive("driver");

monitor mon("monitor");

xogate.a1(a1); xogate.b1(b1); xogate.c1(c1); xogate.d1(d1);

drive.a1(a1); drive.b1(b1); drive.c1(c1);

mon.a1(a1); mon.b1(b1); mon.c1(c1); mon.d1(d1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a1, "a1");

sc_trace(Tf, b1, "b1");

sc_trace(Tf, c1, "c1");

sc_trace(Tf, d1, "d1");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

Output:

- OR

//orgate.h

#include <systemc.h>

SC_MODULE(orgate)

{

sc_in<bool> a1;

sc_in<bool> b1;

sc_in<bool> c1;

sc_out<bool> d1;

void compute_or()

{

d1.write(a1.read() | b1.read() | c1.read());

}

SC_CTOR(orgate)

{

SC_METHOD(compute_or);

sensitive<<a1<<b1<<c1;

}

};

#include <systemc.h>

#include "orgate.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a1, b1,c1, d1;

orgate ogate("orgate");

driver drive("driver");

monitor mon("monitor");

ogate.a1(a1); ogate.b1(b1); ogate.c1(c1); ogate.d1(d1);

drive.a1(a1); drive.b1(b1); drive.c1(c1);

mon.a1(a1); mon.b1(b1); mon.c1(c1); mon.d1(d1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a1, "a1");

sc_trace(Tf, b1, "b1");

sc_trace(Tf, c1, "c1");

sc_trace(Tf, d1, "d1");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a1, b1, c1;

void inputs()

{

a1.write(false);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(true);wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a1 <<b1 <<c1;

}

};

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a1,b1,c1,d1;

void mon()

{

cout<<"Inputs: "<<a1<<b1<<c1<<"Output: "<<d1<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a1<<b1<<c1;

}

};

Output:

- XNOR

//xnorgate.h

#include <systemc.h>

SC_MODULE(xnorgate)

{

sc_in<bool> a1;

sc_in<bool> b1;

sc_in<bool> c1;

sc_out<bool> d1;

void compute_xnor()

{

d1.write(!(a1.read() ^ b1.read() ^ c1.read()));

}

SC_CTOR(xnorgate)

{

SC_METHOD(compute_xnor);

sensitive<<a1<<b1<<c1;

}

};

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a1, b1, c1;

void inputs()

{

a1.write(false);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(true);wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a1 <<b1 <<c1;

}

};

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a1,b1,c1,d1;

void mon()

{

cout<<"Inputs: "<<a1<<b1<<c1<<"Output: "<<d1<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a1<<b1<<c1;

}

};

#include <systemc.h>

#include "xnorgate.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a1, b1,c1, d1;

xnorgate xngate("xnorgate");

driver drive("driver");

monitor mon("monitor");

xngate.a1(a1); xngate.b1(b1); xngate.c1(c1); xngate.d1(d1);

drive.a1(a1); drive.b1(b1); drive.c1(c1);

mon.a1(a1); mon.b1(b1); mon.c1(c1); mon.d1(d1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a1, "a1");

sc_trace(Tf, b1, "b1");

sc_trace(Tf, c1, "c1");

sc_trace(Tf, d1, "d1");

sc_start(30, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

Output:

- AND

//driver.h

#include <systemc.h>

SC_MODULE(driver)

{

sc_out<bool> a1, b1, c1;

void inputs()

{

a1.write(false);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(false);

b1.write(true);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(false);

c1.write(true);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(false);wait(5, SC_NS);

a1.write(true);

b1.write(true);

c1.write(true);wait(5, SC_NS);

}

SC_CTOR(driver)

{

SC_THREAD(inputs);

sensitive <<a1 <<b1 <<c1;

}

};

#include <systemc.h>

#include "andgate.h"

#include "driver.h"

#include "monitor.h"

int sc_main(int argc, char* argv[])

{

sc_signal<bool> a1, b1,c1, d1;

andgate agate("andgate");

driver drive("driver");

monitor mon("monitor");

agate.a1(a1); agate.b1(b1); agate.c1(c1); agate.d1(d1);

drive.a1(a1); drive.b1(b1); drive.c1(c1);

mon.a1(a1); mon.b1(b1); mon.c1(c1); mon.d1(d1);

// waveform

sc_trace_file *Tf;

Tf = sc_create_vcd_trace_file("traces");

sc_trace(Tf, a1, "a1");

sc_trace(Tf, b1, "b1");

sc_trace(Tf, c1, "c1");

sc_trace(Tf, d1, "d1");

sc_start(120, SC_NS);

sc_close_vcd_trace_file(Tf);

return 0;

}

//monitor.h

#include<systemc.h>

SC_MODULE(monitor)

{

sc_in<bool> a1,b1,c1,d1;

void mon()

{

cout<<"Inputs: "<<a1<<b1<<c1<<"Output: "<<d1<<endl;

}

SC_CTOR(monitor)

{

SC_METHOD(mon);

sensitive<<a1<<b1<<c1;

}

};

//andgate.h

#include <systemc.h>

SC_MODULE(andgate)

{

sc_in<bool> a1;

sc_in<bool> b1;

sc_in<bool> c1;

sc_out<bool> d1;

void compute_and()

{

d1.write(a1.read() && b1.read() && c1.read());

}

SC_CTOR(andgate)

{

SC_METHOD(compute_and);

sensitive<<a1<<b1<<c1;

}

};

Output: